EMBBEDED SYSTEMS SPECIALIZATION PROGRAMM

G2 Project #1 LIN

Communications Software Development for Embedded Environments

Feliciano Angulo Angulo Email: se704751@iteso.mx Juan Pablo Garcia Email: se716963@iteso.mx Jesus Íñiguez Email: se717040@iteso.mx

29/03/2018

Embedded Systems Specialization Program

www.sistemasembebidos.iteso.mx/alumnos

I T E S O A.  C., Universidad Jesuita de Guadalajara

Periférico Sur Miguel Gómez Morín #8585, Tlaquepaque, Jalisco, México

Technical Report Number: ESE-O2014-001

® ITESO A.C.

Abstract: This document shows how LIN protocol is implemented using an UART driver in in SAMV71 XPlained evaluation board. Lin and UART implementation are AUTOSAR compliant, whit this implementation is possible to transmit LIN frame through UART, Scope’s captures are included to show LIN frame from Tx line. In first implementation only header was sent, but in this second version a compete master response was implemented.

Keywords: LIN UART AUTOSAR FRAME

# Table of Contents

Table of Contents	ii

1.	Table of Figures	iii

2.	Introduction	1

2.1.	Requirements	1

2.1.1.	Architecture	1

2.1.2.	Implementation	2

3.	Functional Description	4

3.1.	AUTOSAR Implementation	4

3.2.	LIN functions Implementation	6

3.2.1.	State description	8

3.2.2.	IDLE	8

3.2.3.	SEND_BREAK	8

3.2.4.	SEND_SYNC	8

3.2.5.	SEND_PID	8

3.2.6.	SEND_EOT	8

3.2.7.	SEND_RESPONSE	8

4.	Results	8

5.	Conclusions	10

# Table of Figures

Figure 1 AUTOSAR file structure	3

Figure 2 AUTOSAR layers	4

Figure 3 Lin_Init implementation	5

Figure 4 Lin_SendFrame implementation	6

Figure 5 Lin_Isr implementation	6

Figure 6 LIN frame	7

Figure 7 Header capture from Oscilloscope	8

Figure 8 Bit time measure	8

Figure 9 Details of generated LIN frame	9

# Introduction

LIN (Local Interconnect Network) is a serial network protocol used for communication between components in vehicles. LIN implementation is based in UART/SCI hardware, so, when a microcontroller does not have a LIN peripheral integrated but a UART, it can be implemented over this UART easily.

The purpose of this work is to demonstrate the above description, Lin can be implemented over UART. The goal in this second project is to send a complete LIN frame over the UART, specifically a MASTER RESPONSE, a SLAVE response is not part of this goal but it could be easily implemented after have the first.

## Requirements

Provide a SW solution with defined interfaces that will allow to form the LIN Frame through the UART controller available in the SAM V71 controller.

Software must be based on the attached Base Project.

Objective: Implement a functional LIN driver

1.- Based on the Task #1, implement a LIN driver that meets the LIN Specification

2.- Note:

Std_Types.h shall provide the Std_ReturnType data type as an enumeration with the values of:

E_OK: 0

E_NOK: 1

3.- LIN implementation shall follow specification from next file.

### Architecture

The SW must be compliant with the AUTOSAR SW layered architecture, so a File Structure must be provided. This means that (if using UART interfaces) the UART driver interfaces will be at MCAL Layer and LIN interfaces will be at the ECU Abstraction Layer.

File Structure:

../source

->/bsw

->/mcal

->/com

->/uart

(uart files here)

->/ecual

->/com

->/linif

(lin files here)

->/services

->/com

->linNm

(lin files here)

->/system

->/scheduler

(scheduler files here)

Note: In the case that uart driver is used from the provider, file structure shall be kept AS IS.

### Implementation

#### lin_cfg implementation

Lin_Cfg.h –for definition configurable parameters, LIN configuration types: It contains LinChannelType and LinConfigType according to section 5.1.1 LINCONFIG in Lin Specification.pdf file.

Lin_Cfg.c –for configurable parameters: LinChennel[] and LinConfig are created here to configure Lin driver.

#### lin_Types implementation

Consists of types specified within LIN Interface except for configuration types defined above:

LinStateType, LIN_ENHANCED_CS, LinFrameCsModelType, LinFrameResponseType, LinFramePidType, LinFrameDlType, LinPduType according to section 3.2 Types Definition in Lin Specification.pdf file.

#### lin_Nm function implementation

void LinNm_InitData (void)

-This function contains initialization of any available Pdu data to be sent

void LinNm_10ms (void)

-this function is the interface with upper layer to be called when a Lin transmission/reception need to be started.

During this function the system is going to wait for the Slaver Response.

#### linf function implementation

void Lin_Init (const LinConfigType* Config)

- This function will configure the lower layer UART driver

- The function will configure all the logical instances declared linking it with physical channel and its characteristics defined.

- In this function the global variables are set with the parameter gave enabling tx and rx interrupts for the uart hardware.

- Interrupts (NVIC), PIO and clocks shall be configured for each instance.

- Initialize parameter of the LinCtlChnl structure to be used in future operations

- Build and structure with Phy-to-log channels

Std_ReturnType Lin_SendFrame ( uint8_t Channel, LinPduType* PduInfoPtr)

- This function will send a complete LIN frame(MASTER RESPONSE) as per the LIN protocol with the rate define in the Lin_Init function.

- This function shall be asynchronous, i.e. it will trigger the "send command" and will continue its operation without waiting for the header to be completely sent over the bus.

- The frame shall be composed in order of:

- 1. Break = (from 10 to 13 bit times) ideally >= 13 bit time

- 2. Synch = 0x55

- 3. ID = LinPid

-4. Data

-5. Checksum

-In this function the logical channel is sent by parameter and is used to:

-assign the PDU

-calculate the checksum (Lin_CheckSum(Channel)) and parity bits

-Define Slave or Master Task request

In order to support the underlined LIN SW component, a state machine was implemented in next function:

void Lin_Isr(uint8_t PhyChannel)

-Search the logical channel from the physical structure

-This function handle the Sate machine workflow, always is called indirectly by lower layer(uart)

The workflow follows enumeration

typedef enum

{

IDLE = 0,

SEND_BREAK,

SEND_SYNC,

SEND_PID,

SEND_RESPONSE,

READ_RESPONSE,

GET_CHKSUM,

SEND_CHKSUM,

SEND_EOT,

}LinStateType;

The state machine sequence is as follows

IDLE -> SEND_BREAK -> SEND_SYNC -> SEND_PID ->

*SEND_RESPONSE ->SEND_CHKSUM

*READ_RESPONSE ->GET_CHKSUM

- IDLE: No activity.

- SEND_BREAK: Send break process is about to start or in progress.

- SEND_SYNC: Send sync process is about to start or in progress.

- SEND_PID: Send pid and defines if the system has to read some response or just send response.

- SEND_RESPONSE: Send master response data is about to start or in progress, define the workflow to send chksum.

- READ_RESPONSE: read response is about to start or in progress, define the workflow to get chksum.

- SEND_CHECKSUM: Send checksum is about to start or in progress, sends to IDLE.

- GET_CHECKSUM: Get checksum and validate it, sends to IDLE.

# Functional Description

## AUTOSAR description

To make implementation AUTOSAR compliant, the file structure was created according to requirement number 1, it can be seen in figure 1 with required files in specify folder.

Figure 1 AUTOSAR file structure

Interaction between files showed in Figure 1, are based in AUTOSAR layer format, Figure 2 shows such layers, according to this, UART driver is part of MCAL, LIN is implemented as part of ECU Abstraction and LinNm is located in services layer, and a 10 ms task is part of the application layer, all executions are controlled by the scheduler, which is part of services laver as well.

Figure 2 AUTOSAR layers

## LIN functions description

There are 3 main function in Lin to be implemented, Lin_Init, Lin_SendFrame and Lin_Isr,

Figure 3 Lin_Init implementation

As was mentioned before, LIN implementation is in ECU abstraction layer, and according to AUTOSAR, lower interface(MCAL) must be call from here, in this case, needed function from UART driver must be called here, as can be seen in Figure 3, Lin_Init function initialize UART driver, first enabling used GPIO then UART_Configure function is called to configure UART, and at the end interruptions are set. All previous configurations are done for all available Lin channels in the configuration list.

In Scope of current work the channels are defined and sent by parameter, configuring each of them.

They are defined in linif_cfg.c

For our Lin channel one UART channel is used, details are in Table 2.

Table 1 UART I/O Lines details

| Instance | Signal | IO Line | Peripheral |
| --- | --- | --- | --- |
| UART2 | URXD2 | PD25 | C |
| UART2 | UTXD2 | PD26 | C |

Figure 4 Lin_SendFrame implementation

Lin_SendFrame is set to initialize a LIN frame message transmission (see Figure 4), LIN frame transmission is implemented based in a state machine for send each part of the frame.

Before start the state machine, checksum is calculated in case of current lin frame type is a LIN_MASTER_RESPONSE.

Figure 5 Lin_Isr implementation

In Figure 5 can be seen Lin_Isr function, there, is implemented a state machine for control LIN frame states, SEND_BREAK; SEND_SYNC, SEND_PID, SEND_RESPONSE GET_RESPONSE, GET_CHKSUM, READ_CHKSUM. Those states are described in next section.

### State description

### IDLE

No transmission is done here.

### SEND_BREAK

Break is sent here, BREAK consist in a minimum of 13 recessive bits (0), since in a normal UART driver only can be send a max of 8 recessive bits, is necessary to reduce official LIN baud rate to make 8 recessive bits look like at least 13 recessive bits (reduction is done to 5/8 of official baud rate in order to make 8 bit times look like 13 bit times), see figure 7.  Baud rate update is done in this state just before send BREAK_CMD (0x55) value.

Figure 6 SEND_BREAK

Figure 7 Reduce BaudRate Formula

### SEND_SYNC

Here basically is needed send the 0x55 sync byte(SYNC_CMD), but because in previous STATE baud rate was reduced, now baud rate is set to the official LIN baud rate before send SYNC_CMD, details in figure 8.

Figure 8 SEND_SYNC

### SEND_PID

Protected Identifiers is send here just after SEND_SYNC finish, Pid was previously calculated.  Code from Figure 9 shows how after sending it, a decision is done for determinate next state depending of current frame type, LIN_MASTER_RESPONSE or LIN_SLAVE_RESPONSE.

Figure 9 SEND_PID

### SEND_RESPONSE

For send response a Tx counter is used to send as many data as data length(Dl) indicates, once Tx counter is equal to data length next state is defined as SEND_CHKSUM.

Figure 10 SEND_RESPONSE

### SEND_CHKSUM

As checksum value was calculated before here such value is send and next state is defined as IDLE.

Figure 11 SEND_CHKSUM

### READ_RESPONSE

For read response a rx interrupt is enabled and waits to get all the bytes transmited to pass to get_checksum state. The data received is stored in a double pointer for each lin instance

Figure 12 READ_RESPONSE

### GET_RESPONSE

For get response, the state machine waits for the last byte transmitted and verify it

Figure 13 GET_CHKSUM

# Results

The complete LIN frame standard is showed in Figure 13, the target in this work is send such frame complete.

Figure 13 LIN frame

Using the oscilloscope is possible to capture Lin frame for validate that our implementation is sending the complete frame according to the standard. Figure 14 shows the result.

Figure 14 Header capture from Oscilloscope

In figure 9 cursors are measuring the BREAK as 678 microseconds, such measure corresponds to 13 bits in dominant state, since LIN baud rate was set to 19200, 1 bit time is 52 microseconds and 13 bit times are 676 microseconds. Bit time measure is showed in figure 8.

Figure 15 Bit time measure

Figure 9 shows how the header and all its parts are included in generated frame (Break, delimiter, synch byte and PID).

Figure 16 Details of generated LIN frame

# Conclusions

It was demonstrated that LIN frame can be sending using an UART driver since LIN is a Serial Based protocol.

Complete LIN frame was implemented, which is sent by a LIN master node.

Using AUTOSAR architecture standards allows create software with high scalability, modularity and therefore, reusable.

It was demonstrated as well how usage of asynchronous function for send UART messages allows the CPU work in other task while the transmission is in progress.